Effect of Cu/Ce Ratio on Benzene Oxidation over Gold-promoted

Alumina-supported CuO-CeO2

T. Tabakova1*, P. Petrova1, Y. Karakirova1, G. Avdeev2, E. Kolentsova3, L. Ilieva

1Institute of Catalysis, Bulgarian Academy of Sciences, 1113 Sofia, Bulgaria

2Institute of Physical Chemistry, Bulgarian Academy of Sciences, 1113 Sofia, Bulgaria

3 Department of Chemistry and Phytopharmacy, Agricultural University, 4000 Plovdiv, Bulgaria

*Corresponding Author  E-mail address: tabakova@ic.bas.bg

Nowadays, the increased levels and detrimental effects of volatile organic compounds (VOCs) on air quality and human health have become a serious global concern from environmental point of view. Among variety of technologies, complete oxidation to CO2 and water at relatively low temperature is one of the most effective and environmentally-friendly approach for VOCs removal. Rational design of catalytic materials with high effectiveness in the destructive elimination of VOCs is one of the scientific challenges. The aim of the present work is to study complete benzene oxidation (CBO) over Al2O3-supported CuO-CeO2 mixed oxide promoted by gold. The investigation is focused on the effect of various Cu/Ce molar ratios (1:5 or 2:1) on benzene conversion. Benzene molecule is the most stable among the aromatic hydrocarbons with toxic and often carcinogenic effect owing to the symmetry and stability of the benzene ring.

Catalytic activity was greatly influenced by sample composition. Catalytic tests demonstrated that the Cu-Ce 1:5 configuration outperformed the catalytic behavior of the single-component counterparts in the whole temperature interval.

Promotion by gold provided additional benefits to the benzene conversion. Gold on alumina-supported binary oxide catalysts exhibited a substantially higher activity than gold supported on monometallic oxides. Our findings revealed a crucial role of highly dispersed CuOx for benzene oxidation activity. Au/Cu-Ce 1:5 achieved stable total oxidation to CO2 and water by 94 % benzene conversion at 250 C, thus implying the potential of this composition in developing efficient catalytic materials for atmospheric pollutant abatement.

Acknowledgment This work was supported by the European Regional Development Fund within the Operational Programme “Science and Education for Smart Growth 2014 - 2020”, Project CoE “National center of mechatronics and clean technologies“ BG05M2OP001-1.001-0008".

Tatyana Tabakova is a Professor of Chemical Kinetics and Catalysis at the Institute of Catalysis – Bulgarian Academy of Sciences since 2012. She received her PhD degree in 1994 from the Institute of Catalysis. Her research interests involve various aspect of heterogeneous catalysis, such as application of different methods for synthesis of new materials and catalysts, investigation of the relationship between structure and catalytic activity of catalysts with fundamental and practical applications. Currently, she is interested in preparation and study of gold catalysts supported on different oxides for reaction with environmental impact, in particular clean hydrogen production via water-gas shift and selective CO oxidation reactions, volatile organic compounds and CO oxidation. She has published more than 100 peer-reviewed papers (bringing 5000+ citations and forming an h-index of 39), coordinated and participated in many projects (COST, NATO, bilateral projects with Universities and research Institutes from Italy, Greece, Spain, Poland, Belgium, China, etc).