Development and Evaluation of Mannose Decorated Bovine Serum Albumin Coated Superparamagnetic Iron Oxide Nanoparticles (MnBSA-SPIONs) for Liver Imaging.

Chetan Nehate,1,2 Veena koul,1,2

1 Centre for Biomedical Engineering, Indian Institute of Technology Delhi, Hauz Khas, New Delhi, India.

2 Biomedical Engineering Unit, All India Institute of Medical Sciences, Ansari Nagar, New Delhi, India.

Abstract:

Liver diseases such as viral hepatitis, cirrhosis and hepatocellular carcinoma, are common in clinical practice with high morbidity and mortality worldwide. Hepatocyte selective imaging contrast agents can provide useful information for evaluating hepatic function non-invasively in vivo. Recently, superparamagnetic iron oxide nanocrystals (SPIONs) have been widely explored as magnetic resonance imaging (MRI) contrast agents in biomedical research [1]. The mannose receptor is a transmembrane protein expressed on the surface of macrophages, including Kupffer cells and endothelial cells [2]. We aimed to target these receptors for MRI imaging of liver. Oleic acid coated SPIONs were developed with high crystallinity and magnetization by modified thermal decomposition method. The SPIONs were further made water soluble by ligand exchange with (3-aminopropyl)triethoxysilane (APTS). The size of APTS coated SPIONs was fond to be 9.5 nm. These hydrophilic SPIONs were encapsulated in albumin nanoparticles and further mannosylated by schiff base formation in acetate buffer pH 4. The mannose conjugation was confirmed by decrease in free amino groups on nanoparticles with TNBS assay. The MnBSA-SPIONs were characterized by DLS, TEM, SEM, XRD, AAS etc. The uptake of MnBSA-SPIONs was monitored in J774 cell line by prussian blue staining at different time intervals. The biocompatibility of MnBSA-SPIONs was tested in J774 cell line by MTT assay and was found to be biocompatible. The T2 relaxivity of nanoparticles was determined in phantom agar gels and was found to be 394 mM-1S-1. The invivo MRI for liver imaging is currently underway.

Keywords: MRI, SPIONs, BSA, Relaxivity.

Figure 1: Characterization of nanoparticles by high resolution transmission electron microscopy (A) APTS-SPIONs (B) MnBSA-SPIONs

References:

- Barrow, M., et al., Chemical Society Reviews, 2015. 44(19):6733-6748.

- Benedicto, A., et al., Cancer Research, 2015. 75(1 Supplement): p. B10-B10.