Tunable and reconfigurable integrated filter based on long-period waveguide grating

A tunable and reconfigurable integrated filter based on long-period waveguide grating is proposed and studied in this paper. It consists of a long-period waveguide grating fabricated on Lithium Niobate substrate and multiple electrode pairs deposited on both sides of gratings. The performances of the filter are analyzed theoretically and measured experimentally. The spectrum of the filter is tuned by adding same voltage on all electrode pairs and the spectrum function is reconfigured by applying different voltage on different electrode pairs due to the electro-optic effect in Lithium Niobate.